TEAM 05 - LOAD & RUN - FINAL DELIVERABLE DOCUMENT

XR & AI Genomics Hackathon 2026 | IGH Redeemer's University

Project Pipeline: From Genome to Impact - PCR VR Simulation

Stage: 05 - Load & Run

1. OBJECTIVE

As per Project Pipeline Slide Page 8: Simulate loading a prepared 96-well plate into a real-time PCR cycler, set the thermal program (35 cycles, 3 temperatures), visualize denaturation → annealing → extension, and read the real-time amplification result. Total run ∼2hrs, 2³⁵ copies.

2. INPUT FROM TEAM 04

We receive: Sealed 96-well plate, centrifuged, on cold block.

- Layout: 96 wells = Master Mix 24 + PTC 8 + Samples 52 + NTC 8 + Cross-check 4

- For this demo run: 26 samples + 1 Positive Control + 1 Negative Control (NTC) = 28 reactions

3. DELIVERABLES BREAKDOWN

D1: LOAD - Loading the Plate

- Visual check: optical film sealed, no bubbles, A1 orientation

- Plate centrifuge interaction: 1000rpm x 1min spin to remove bubbles

- Equipment: Pre-filled plate, Plate centrifuge

D2: MACHINE - PCR Machine Interaction

- Boot machine (power button)

- Open drawer/lid animation (using Biobase Real-Time PCR - drawer type as reference)

- Slide plate in - A1 must align to block A1 - else error popup "Invalid orientation"

- Close drawer/lid - Lid temp 105°C

- Equipment: Biobase Real-Time PCR Machine

D3: PROGRAM - Setting PCR Conditions

Player sets on machine touchscreen:

- Reaction Volume: 20µL

- Lid Temp: 105°C

- Fluorophore: SYBR Green

- Thermal Profile:

- Hold: 95°C for 02:00 min

- Cycling: 35 cycles of 95°C x 15s → 60°C x 30s → 72°C x 30s

- Melt Curve: 65°C - 95°C

- Method: Real-time fluorescence read at 60°C annealing step

D4: VISUALIZ - PCR Cycle Visualization

- Counter on screen: Cycle 1/35 → 35/35, Progress 0-100%

- Inside view animation: 95°C RED (denaturation - strands unzip), 60°C BLUE (annealing - primers bind), 72°C GREEN (extension - DNA builds)

- Loop x35

D5: COMPLETE - PCR Completion State

- Machine beep + Popup: RUN COMPLETE - 35/35 Cycles - Status: PASSED

- Run time simulated: 1hr 12min → compressed to 30sec in VR

D6: RESULTS - Real-Time PCR Results (ON MACHINE SCREEN)

- Screen displays:

- Amplification Curve: Fluorescence (RFU) vs Cycle (0-35)

- Ct Values Table: 26 Samples (S1-S26) with Ct 22.4 - 26.1 = POSITIVE, POS CTRL 18.5 = POSITIVE, NTC = NEGATIVE

- Validation Rule: POS must amplify, NTC must NOT amplify. If NTC amplifies = INVALID/CONTAMINATION

- Final call: Threshold line, Ct <35 = Positive

4. EQUIPMENT LIST FOR UNITY/BLENDER

Biobase Real-Time PCR, 96-well plate (A1 marked), Plate centrifuge, Nitrile gloves, Pipettes (background), VR Headset interaction, Computer touchscreen UI

5. FAIL STATES / MENTOR DIALOGUE

- Wrong A1 placement: "Check orientation - A1 must match block A1"

- No spin: "Bubbles detected - fluorescence will be noisy"

- NTC amplifies: "Contamination detected - Run Invalid"

- Correct action: Label glows + "Great! Plate loaded correctly"

Reference Image: Biobase drawer model with results on screen - 35 cycles, 26 samples + controls attached.